# Fire progression in Sumatra and Kalimantan, Indonesia, 2001-2024

Author: Sarah Jasmine Riadi & Manzhu Yu, Pennsylvania State University

# Data structure and variables

This dataset contains an organized collection of fire events with two types of shapefiles: 1) points and 2) grid polygons. Each shapefile type captures a distinct spatial representation of the same fire events. Both files share a common fire_id field, which serves as the primary key for joining records across files. For example, one can retrieve the daily spread sequence from the point file and the environmental attributes from the grid file, all linked through fire_id.

Table 1. Fire point variables

| Variable name | Unit | Data Type | Description |
| --- | --- | --- | --- |
| index_af | - | Integer | Unique identifier of each MODIS active fire detection |
| LATITUDE | Degrees | Float | Latitude of MODIS AF points |
| LONGITUDE | Degrees | Float | Longitude of MODIS AF points |
| BRIGHTNESS | K | Float | Brightness temperature at Band 21/22 (mid-infrared) |
| BRIGHT_T31 | K | Float | Brightness temperature at Band 31 (thermal infrared) |
| FRP | MW | Float | Fire radiative power |
| CONFIDENCE | % | Integer | MODIS detection confidence level (retained ≥ 60%) |
| grid_id | - | Integer | Unique identifier of the intersecting MODIS 500 m grid cell |
| lon | Degrees | Float | Longitude of the associated MODIS BA grid centroid |
| lat | Degrees | Float | Latitude of the associated MODIS BA grid centroid |
| fire_id | - | Integer | Unique identifier for the parent fire event; links to hull and grid files |
| af_type | - | String | Ignition identifier (ig) Active fire role within the event: “ig” = ignition point (first detection day); “non_ig” = subsequent detection |
| ACQ_DATE | YYYY-MM-DD | Datetime | Acquisition date of active fire detection |
| ACQ_TIME | HH:MM | Datetime | Acquisition time of active fire detection |
| ACQ_DATETI | YYYY-MM-DD HH:MM | Datetime | Combined acquisition date and time |
| year | - | Datetime | Year from ACQ_DATE. |
| month | - | Datetime | Month from ACQ_DATE |
| province | - | String | Sumatra/Kalimantan region in which the fire point falls |
| burn_durat | Days | Integer | The total duration in days of the whole one fire event |
| min_date_b | YYYY-MM-DD | Datetime | Earliest burn date recorded in the MODIS MCD64A1 burned area product within the spatiotemporal extent of this fire event |
| LON_M | - | Float | Easting coordinate in meters (Albers Equal Area custom projection) |
| LAT_M | - | Float | Northing coordinate in meters Albers Equal Area custom projection) |
| ros_point | meters/day | Float | Point-level ROS: distance from nearest previous-day detection divided by elapsed days |
| dist_point | meters (m) | Float | Distance from this detection to the nearest detection on the previous observation day |
| days_diff | Days | Integer | Elapsed days between this detection and the nearest detection on the previous observation day |
| dist_event | meters (m) | Float | Cumulative travel distance of fire detections from first ignition to this point |
| cumm_days | Days | Integer | Cumulative days elapsed since first ignition to this detection |
| fire_durat | Days | Integer | Total duration of the parent fire event in days (last detection date minus first detection date) |
| ros_avg_cu | meters/day | Float | Cumulative average ROS from ignition to this point: dist_event / cumm_days |
| ros_avg_du | meters/day | Float | Event-average ROS: dist_event / fire_durat |
| step1_id | - | Integer | The index_af of previous detection (point A in the spread pair) |
| step2_id | - | Integer | The index_af of this detection (point B in the spread pair) |
| geometry | - | Geometry | Point geometry |

Table 2 Fire grid variables

| Variable name | Unit | Data Type | Description |
| --- | --- | --- | --- |
| grid_id | - | Integer | Unique identifier of the MODIS 500 m grid cell |
| lon | Degrees | Float | Longitude of the grid cell centroid |
| lat | Degrees | Float | Latitude of the grid cell centroid |
| fire_id | - | Integer | Unique identifier for the parent fire event; links to point and hull files |
| year | - | Datetime | Year extracted from nearest active fire acquisition date |
| month | - | Datetime | Month extracted from nearest active fire acquisition date |
| af_count | - | Integer | Number of MODIS active fire detections intersecting this grid cell |
| area_km2 | km2 | Float | Total fire event area computed from the buffered convex hull |
| buffer_m | meters | Integer | Perimeter buffer applied to the convex hull (250 m for all events) |
| date_min | YYYY-MM-DD | Datetime | First detection date within the fire event |
| date_max | YYYY-MM-DD | Datetime | Last detection date within the fire event |
| af_type | - | String | Identifier of active fire intersection with grids (ig: ignition, non ig: non ignition) |
| af_ig_ | - | Float | Identifier of active fire intersection with grids with binary value (ig: 1, non ig: 0) |
| nearest_po | - | Geometry | Nearest active fire point from the grid |
| n_ACQ_DATE | YYYY-MM-DD | Datetime | Acquisition date of nearest active fire to this grid cell |
| n_ACQ_TIME | HH-MM | Datetime | Acquisition time of nearest active fire to this grid cell |
| distance_t | meters | Float | Distance to nearest active fire point from the grid centroid |
| nearest__1 | Degrees | Float | Latitude of nearest active fire point |
| nearest__2 | Degrees | Float | Longitude of nearest active fire point |
| ACQ_DATE_C | YYYY-MM-DD | Datetime | ACQ_DATE column as a cftime (time units of Climate and Forecasting metadata) |
| ndvi | - | Float | NDVI value from the nearest pre-fire MODIS MOD13A1 16-day composite |
| evi | - | Float | EVI value from nearest pre-fire MODIS MOD13A1 16-day composite |
| ndvi_date | YYYY-MM-DD | Datetime | NDVI / EVI acquisition date |
| elevation | m | Float | Elevation above sea level from NASADEM |
| mapbiomas | - | Integer | MapBiomas Collection 4 LULC class code assigned to this grid cell |
| lulc_code | - | List | MapBiomas Collection 4 LULC class code assigned to this grid cell |
| major_dail | - | Float | Dominant LULC class code burning on the acquisition day of this grid cell |
| major_da_1 | km2 | Float | Area of the dominant daily LULC class |
| major_da_2 | % | Float | Fractional area of the dominant daily LULC class as a percentage of total daily burn area |
| major_lulc | - | Integer | Dominant LULC class code across the full fire event |
| major_km2 | km2 | Float | Area of the dominant event-level LULC class |
| major_pct | % | Float | Fractional area of the dominant event-level LULC class |
| total_km2 | km2 | Float | Total fire event area computed from grid cell count |
| burn_durat | Days | Integer | Total burn duration of one fire event |
| ros_km2_da | km2/day | Float | ROS event-level of total area/burn duration |
| first_lulc | - | Integer | Dominant LULC class code on the first day of the fire event |
| last_lulc | - | Integer | Dominant LULC class code on the last day of the fire event |
| pair_end | - | Integer | Encoded first-to-last LULC transition: (first_lulc × 100) + last_lulc |
| s_seq_3 | - | List | Simplified three-point LULC progression sequence |
| s_seq_5 | - | List | Simplified five-point LULC progression sequence |
| conv_preci | m | Float | Convective precipitation (ERA5) |
| conv_rain | mm/hr | Float | Convective rain rate (ERA5) |
| d2m | C | Float | 2m dewpoint temperature (ERA5) |
| dir_deg | Degree | Float | Direction of wind, from North, clockwise |
| ev_h | m | Float | Evaporation (ERA5) |
| gust | m/s | Float | Instantaneous 10m wind gust (ERA5) |
| mslp_pa | Pa | Float | Mean sea level pressure (ERA5) |
| pev_hourly | M | Float | Potential evaporation (ERA5) |
| rh | % | Float | Relative humidity, derived from d2m and t2m |
| sp_pa | Pa | Float | Surface pressure (ERA5) |
| sst | C | Float | Sea surface temperature (ERA5) |
| t2m | C | Float | 2m temperature (ERA5) |
| tp_hourly | m | Float | Total precipitation (ERA5) |
| vpd | kPa | Float | Vapor pressure deficit, derived from d2m and t2m |
| wind_speed | m/s | Float | Wind speed |
| geometry | - | Geometry | Grid cell polygon geometry |